Here’s the extracted MPU-6050 accelerometer code from your STM32 project, cleaned up so your friend can copy and paste it directly. The source came from your uploaded file

1. ADD THESE #define VALUES

Add these in:

/* USER CODE BEGIN PD */

section.

// ===== MPU6050 ACCELEROMETER =====
#define MPU_ADDR             (0x68 << 1)

#define MPU_REG_WHO_AM_I     0x75
#define MPU_REG_PWR_MGMT_1   0x6B
#define MPU_REG_ACCEL_CONFIG 0x1C
#define MPU_REG_ACCEL_XOUT_H 0x3B

#define MPU_REG_SMPLRT_DIV   0x19
#define MPU_REG_CONFIG       0x1A
#define MPU_REG_PWR_MGMT_2   0x6C

#define ACCEL_LSB_PER_G        16384.0f

#define UNSAFE_THRESHOLD_G     0.50f
#define IMPACT_THRESHOLD_G     1.50f

#define ACCEL_UPDATE_PERIOD_MS 100U

// Calibration values
#define ACCEL_X_BIAS_G         0.1000f
#define ACCEL_X_SCALE          1.0204f

#define ACCEL_Y_BIAS_G        -0.0150f
#define ACCEL_Y_SCALE          1.0050f

#define ACCEL_Z_BIAS_G         0.0150f
#define ACCEL_Z_SCALE          0.9950f

2. ADD THESE VARIABLES

Add these in:

/* USER CODE BEGIN PV */

section.

/* ===== MPU6050 ACCELEROMETER ===== */

uint8_t mpu_ok = 0;

int16_t accel_x_raw = 0;
int16_t accel_y_raw = 0;
int16_t accel_z_raw = 0;

float accel_x_g = 0.0f;
float accel_y_g = 0.0f;
float accel_z_g = 0.0f;

float accel_abs_g = 0.0f;

float accel_x_offset_g = 0.0f;
float accel_y_offset_g = 0.0f;
float accel_z_offset_g = 0.0f;

float accel_x_dyn_g = 0.0f;
float accel_y_dyn_g = 0.0f;
float accel_z_dyn_g = 0.0f;

uint32_t last_accel_update_ms = 0;

// Alarm flags
volatile uint8_t unsafe_alarm = 0;
volatile uint8_t impact_alarm = 0;

3. ADD THESE FUNCTION PROTOTYPES

Add these in:

/* USER CODE BEGIN PFP */

section.

uint8_t MPU6050_Init(void);
uint8_t MPU6050_ReadAccel(void);
void MPU6050_CalibrateStill(void);
void UpdateMPUAlarms(void);

4. ADD THESE FUNCTIONS

Add these in:

/* USER CODE BEGIN 0 */

section.

Write Register

uint8_t MPU6050_WriteReg(uint8_t reg, uint8_t data)
{
    return (HAL_I2C_Mem_Write(&hi2c1,
                              MPU_ADDR,
                              reg,
                              I2C_MEMADD_SIZE_8BIT,
                              &data,
                              1,
                              100) == HAL_OK);
}

Read Register

uint8_t MPU6050_ReadReg(uint8_t reg, uint8_t *data)
{
    return (HAL_I2C_Mem_Read(&hi2c1,
                             MPU_ADDR,
                             reg,
                             I2C_MEMADD_SIZE_8BIT,
                             data,
                             1,
                             100) == HAL_OK);
}

Read Multiple Bytes

uint8_t MPU6050_ReadBytes(uint8_t reg, uint8_t *data, uint8_t len)
{
    return (HAL_I2C_Mem_Read(&hi2c1,
                             MPU_ADDR,
                             reg,
                             I2C_MEMADD_SIZE_8BIT,
                             data,
                             len,
                             100) == HAL_OK);
}

MPU6050 Init

uint8_t MPU6050_Init(void)
{
    uint8_t who = 0;

    HAL_Delay(100);

    if(!MPU6050_ReadReg(MPU_REG_WHO_AM_I, &who))
    {
        mpu_ok = 0;
        return 0;
    }

    if(who != 0x68)
    {
        mpu_ok = 0;
        return 0;
    }

    MPU6050_WriteReg(MPU_REG_PWR_MGMT_1, 0x80);
    HAL_Delay(100);

    MPU6050_WriteReg(MPU_REG_PWR_MGMT_1, 0x01);
    HAL_Delay(20);

    MPU6050_WriteReg(MPU_REG_PWR_MGMT_2, 0x00);
    MPU6050_WriteReg(MPU_REG_CONFIG, 0x03);
    MPU6050_WriteReg(MPU_REG_SMPLRT_DIV, 9);

    // ±2g range
    MPU6050_WriteReg(MPU_REG_ACCEL_CONFIG, 0x00);

    mpu_ok = 1;

    return 1;
}

Read Accelerometer

uint8_t MPU6050_ReadAccel(void)
{
    uint8_t data[6];

    if(!mpu_ok)
        return 0;

    if(!MPU6050_ReadBytes(MPU_REG_ACCEL_XOUT_H, data, 6))
    {
        mpu_ok = 0;
        return 0;
    }

    accel_x_raw = (int16_t)((data[0] << 8) | data[1]);
    accel_y_raw = (int16_t)((data[2] << 8) | data[3]);
    accel_z_raw = (int16_t)((data[4] << 8) | data[5]);

    accel_x_g =
        ((((float)accel_x_raw) / ACCEL_LSB_PER_G)
        - ACCEL_X_BIAS_G)
        * ACCEL_X_SCALE;

    accel_y_g =
        ((((float)accel_y_raw) / ACCEL_LSB_PER_G)
        - ACCEL_Y_BIAS_G)
        * ACCEL_Y_SCALE;

    accel_z_g =
        ((((float)accel_z_raw) / ACCEL_LSB_PER_G)
        - ACCEL_Z_BIAS_G)
        * ACCEL_Z_SCALE;

    accel_x_dyn_g = accel_x_g - accel_x_offset_g;
    accel_y_dyn_g = accel_y_g - accel_y_offset_g;
    accel_z_dyn_g = accel_z_g - accel_z_offset_g;

    float ax = fabsf(accel_x_dyn_g);
    float ay = fabsf(accel_y_dyn_g);
    float az = fabsf(accel_z_dyn_g);

    accel_abs_g = ax;

    if(ay > accel_abs_g)
        accel_abs_g = ay;

    if(az > accel_abs_g)
        accel_abs_g = az;

    return 1;
}

Calibration

void MPU6050_CalibrateStill(void)
{
    float sx = 0.0f;
    float sy = 0.0f;
    float sz = 0.0f;

    uint16_t good = 0;

    for(uint16_t i = 0; i < 100; i++)
    {
        if(MPU6050_ReadAccel())
        {
            sx += accel_x_g;
            sy += accel_y_g;
            sz += accel_z_g;

            good++;
        }

        HAL_Delay(10);
    }

    if(good > 0)
    {
        accel_x_offset_g = sx / good;
        accel_y_offset_g = sy / good;
        accel_z_offset_g = sz / good;
    }
}

Alarm Logic

void UpdateMPUAlarms(void)
{
    if(!mpu_ok)
    {
        unsafe_alarm = 0;
        impact_alarm = 0;
        return;
    }

    if(accel_abs_g > IMPACT_THRESHOLD_G)
    {
        impact_alarm = 1;
        unsafe_alarm = 1;
    }
    else if(accel_abs_g > UNSAFE_THRESHOLD_G)
    {
        impact_alarm = 0;
        unsafe_alarm = 1;
    }
    else
    {
        impact_alarm = 0;
        unsafe_alarm = 0;
    }
}

5. INITIALISE THE MPU6050

Add this in main() after:

MX_I2C1_Init();

and after OLED init is fine.

// ===== MPU6050 INIT =====
mpu_ok = MPU6050_Init();

if(mpu_ok)
{
    MPU6050_CalibrateStill();
}

6. ADD THIS TO THE while(1) LOOP

if((HAL_GetTick() - last_accel_update_ms) >= ACCEL_UPDATE_PERIOD_MS)
{
    last_accel_update_ms = HAL_GetTick();

    if(MPU6050_ReadAccel())
    {
        UpdateMPUAlarms();
    }
}

7. REQUIRED STM32CubeMX SETTINGS

Your friend must configure:

I2C1

- Mode: I2C

- Speed: 100 kHz

- Addressing: 7-bit

MPU6050 Wiring

| MPU6050 | STM32 |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| SDA | I2C1 SDA |
| SCL | I2C1 SCL |

8. REQUIRED INCLUDE

Add this near the top:

#include <math.h>

because the code uses:

fabsf()

9. OPTIONAL UART DEBUG

To print accelerometer readings:

char msg[64];

sprintf(msg,
        "X:%1.2f Y:%1.2f Z:%1.2f\r\n",
        accel_x_g,
        accel_y_g,
        accel_z_g);

HAL_UART_Transmit(&huart2,
                  (uint8_t*)msg,
                  strlen(msg),
                  100);

You can place that inside the while(1) loop after MPU6050_ReadAccel().

Source extracted from your uploaded STM32 project file.