/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Main program body

******************************************************************************

* @attention

*

* Copyright © 2026 STMicroelectronics.

* All rights reserved.

*

* This software is licensed under terms that can be found in the LICENSE file

* in the root directory of this software component.

* If no LICENSE file comes with this software, it is provided AS-IS.

*

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include “main.h”

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

#include <stdio.h>

#include <string.h>

#include <stdlib.h>

#include “ssd1306.h”

#include “ssd1306_fonts.h”

/* USER CODE END Includes */

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

Void FlashD2LEDS(GPIO_TypeDef *port1, uint16_t pin1,float frequency);

Void FlashD5LEDS(GPIO_TypeDef *port2, uint16_t pin2,float frequency);

/* USER CODE END PD */

/* Private macro -------------------------------------------------------------*/

/* USER CODE BEGIN PM */

/* USER CODE END PM */

/* Private variables ---------------------------------------------------------*/

I2C_HandleTypeDef hi2c1;

RTC_HandleTypeDef hrtc;

TIM_HandleTypeDef htim3;

TIM_HandleTypeDef htim4;

UART_HandleTypeDef huart2;

/* USER CODE BEGIN PV */

Uint32_t IC_Val1 = 0;

Uint32_t IC_Val2 = 0;

Uint8_t Is_First_Captured = 0;

Uint32_t CurrentTick = 0;

// Debounce trackers for each specific button

Uint32_t LastTick_S1_PB10 = 0; // UP button

Uint32_t LastTick_S2_PA8  = 0; // LEFT button

Uint32_t LastTick_S3_PB4  = 0; // MIDDLE button

Uint32_t LastTick_S4_PB5  = 0; // RIGHT button

Uint32_t LastTick_S5_PA9  = 0; // DOWN button

// Button Action Flags (Set in ISR, processed in main)

Volatile uint8_t flag_S1 = 0;

Volatile uint8_t flag_S2 = 0;

Volatile uint8_t flag_S3 = 0;

Volatile uint8_t flag_S4 = 0;

Volatile uint8_t flag_S5 = 0;

// Initialize to 1 so they turn on immediately at startup (Demo 1 requirement)

Uint8_t D2On = 0;

Uint8_t D3On = 0;

Uint8_t D4On = 0;

Uint8_t D5On = 0;

// --- Temperature Sensor Tracking Variables ---

Volatile uint32_t pulse_count = 0;

Volatile uint32_t last_capture_time = 0;

Volatile float current_temperature = -50.0f;

Volatile uint8_t temp_ready = 0;

// Alarm and Telemetry Tracking

Volatile uint8_t alarmEnabled = 1;

Volatile uint8_t statRequested = 0;

Uint8_t rxByte;

Uint8_t rxBuffer[64]; // Increased from 10 to 64 to fit long commands!

Uint8_t rxIndex = 0;

// Sensor Data

Float distance = 0.0f;

Float x_accel = 0.0f, y_accel = 0.0f, z_accel = 0.0f;

Float light = 0.0f;

Uint8_t unsafe = 0, impact = 0, low_light = 0, proximity = 0, high_temp = 0;

Float gps_lat = 0.0f, gps_lon = 0.0f;

Char datetime[30] = “”; // Live RTC string

// Define the menu pages from Table 5

// Define the menu pages from Table 5

Typedef enum {

MENU_TOP_MEASUREMENTS = 10,

MENU_MEASUREMENTS_1_1,

MENU_MEASUREMENTS_1_2,

MENU_MEASUREMENTS_1_3,

MENU_MEASUREMENTS_1_4,

MENU_MEASUREMENTS_1_5,

MENU_TOP_DATA_ENTRY = 20,

MENU_DATA_ENTRY_2_1,

MENU_DATA_ENTRY_2_2,

MENU_DATA_ENTRY_2_3,

MENU_TOP_DIAGNOSTICS = 30,

MENU_DIAGNOSTICS_3_1,

MENU_DIAGNOSTICS_3_2,

MENU_DIAGNOSTICS_3_3,

MENU_DIAGNOSTICS_3_4

} MenuState_t;

MenuState_t currentMenu = MENU_TOP_MEASUREMENTS;

// Data Entry Variables

Uint8_t is_editing_data = 0;

Char fuel_input_str[10] = “”;

Uint8_t fuel_idx = 0;

// Keypad Tracking Variables

Uint32_t LastTick_Keypad = 0;

Volatile uint8_t flag_Keypad = 0;

Volatile char lastKeyPressed = ‘\0’;

Volatile uint16_t active_column_pin = 0;

// --- Odometer Variables ---

Uint8_t is_editing_odo = 0;

Char odo_input_str[10] = “”;

Uint8_t odo_idx = 0;

// --- Data Logging Variables ---

Uint8_t is_logging_enabled = 0; // 0 = OFF, 1 = ON

// UART Command Variables

Volatile uint8_t rfeRequested = 0;

Volatile float fuel_liters = 0.0f;

Volatile float odo_km = 0.0f;

Volatile uint8_t force_prox = 0;

Volatile uint8_t force_temp = 0;

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

Void SystemClock_Config(void);

Static void MX_GPIO_Init(void);

Static void MX_USART2_UART_Init(void);

Static void MX_TIM4_Init(void);

Static void MX_TIM3_Init(void);

Static void MX_I2C1_Init(void);

Static void MX_RTC_Init(void);

/* USER CODE BEGIN PFP */

Void HCSR04_Trigger(void);

Char read_keypad_matrix(uint16_t column_pin);

/* USER CODE END PFP */

/* Private user code ---------------------------------------------------------*/

/* USER CODE BEGIN 0 */

/* USER CODE END 0 */

/**

* @brief  The application entry point.

* @retval int

*/

Int main(void)

{

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

/* MCU Configuration--------------------------------------------------------*/

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* USER CODE BEGIN Init */

/* USER CODE END Init */

/* Configure the system clock */

SystemClock_Config();

/* USER CODE BEGIN SysInit */

/* USER CODE END SysInit */

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_USART2_UART_Init();

MX_TIM4_Init();

MX_TIM3_Init();

MX_I2C1_Init();

MX_RTC_Init();

/* USER CODE BEGIN 2 */

Char msg[]= “*27434435#\n”;

HAL_Delay(100);

HAL_UART_Transmit(&huart2, (uint8_t*)msg, strlen(msg), 10);

// Start listening for the first UART byte

HAL_UART_Receive_IT(&huart2, &rxByte, 1);

// Start the TIM4 hardware timer in Interrupt mode to listen to PB6

HAL_TIM_IC_Start_IT(&htim4, TIM_CHANNEL_1);

// --- ADD THIS OLED TEST CODE ---

Ssd1306_Init(); // Wakes up the OLED

Ssd1306_Fill(Black); // Clears the screen

// Set cursor to X=0, Y=0 (Top Left) and print

Ssd1306_SetCursor(0, 0);

Ssd1306_WriteString(“Only way is up!”, Font_7x10, White);

// Push the data from the STM32 memory to the actual screen

Ssd1306_UpdateScreen();

// -------------------------------

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

While (1)

{

If (flag_S1) {

If ((HAL_GetTick() – LastTick_S1_PB10) > 50) {

Flag_S1 = 0;

If (HAL_GPIO_ReadPin(GPIOB, GPIO_PIN_10) == GPIO_PIN_SET) {

// Top Menu Cycle

If (currentMenu == MENU_TOP_MEASUREMENTS) currentMenu = MENU_TOP_DIAGNOSTICS;

Else if (currentMenu == MENU_TOP_DATA_ENTRY) currentMenu = MENU_TOP_MEASUREMENTS;

Else if (currentMenu == MENU_TOP_DIAGNOSTICS) currentMenu = MENU_TOP_DATA_ENTRY;

// Measurement Sub-Menu Cycle

Else if (currentMenu == MENU_MEASUREMENTS_1_1) currentMenu = MENU_MEASUREMENTS_1_5;

Else if (currentMenu > MENU_MEASUREMENTS_1_1 && currentMenu <= MENU_MEASUREMENTS_1_5) currentMenu--;

// Data Entry Sub-Menu Cycle

Else if (currentMenu == MENU_DATA_ENTRY_2_1) currentMenu = MENU_DATA_ENTRY_2_3;

Else if (currentMenu > MENU_DATA_ENTRY_2_1 && currentMenu <= MENU_DATA_ENTRY_2_3) currentMenu--;

// Diagnostics Sub-Menu Cycle

Else if (currentMenu == MENU_DIAGNOSTICS_3_1) currentMenu = MENU_DIAGNOSTICS_3_4;

Else if (currentMenu > MENU_DIAGNOSTICS_3_1 && currentMenu <= MENU_DIAGNOSTICS_3_4) currentMenu--;

}

}

}

If (flag_S2) {

If ((HAL_GetTick() – LastTick_S2_PA8) > 50) {

Flag_S2 = 0;

If (HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_8) == GPIO_PIN_SET) {

// Safely exit ANY sub-menu back to its parent

If (currentMenu >= MENU_MEASUREMENTS_1_1 && currentMenu <= MENU_MEASUREMENTS_1_5) currentMenu = MENU_TOP_MEASUREMENTS;

Else if (currentMenu >= MENU_DATA_ENTRY_2_1 && currentMenu <= MENU_DATA_ENTRY_2_3) currentMenu = MENU_TOP_DATA_ENTRY;

Else if (currentMenu >= MENU_DIAGNOSTICS_3_1 && currentMenu <= MENU_DIAGNOSTICS_3_4) currentMenu = MENU_TOP_DIAGNOSTICS;

}

}

}

If (flag_S3) {

If ((HAL_GetTick() – LastTick_S3_PB4) > 50) {

Flag_S3 = 0;

If (HAL_GPIO_ReadPin(GPIOB, GPIO_PIN_4) == GPIO_PIN_SET) {

// 1. Toggle Fuel Edit Mode

If (currentMenu == MENU_DATA_ENTRY_2_1) {

Is_editing_data = !is_editing_data;

If (is_editing_data) {

Memset(fuel_input_str, 0, sizeof(fuel_input_str));

Fuel_idx = 0;

}

}

// 2. Toggle Odometer Edit Mode

Else if (currentMenu == MENU_DATA_ENTRY_2_2) {

Is_editing_odo = !is_editing_odo;

If (is_editing_odo) {

Memset(odo_input_str, 0, sizeof(odo_input_str));

Odo_idx = 0;

}

}

// 3. Normal Alarm Behavior (if not on an edit page)

Else {

alarmEnabled = 1;

D2On = 0; D3On = 0; D4On = 0; D5On = 0;

Proximity = 0; high_temp = 0;

Force_prox = 0;

Force_temp = 0;

}

}

}

}

If (flag_S4) {

If ((HAL_GetTick() – LastTick_S4_PB5) > 50) {

Flag_S4 = 0;

If (HAL_GPIO_ReadPin(GPIOB, GPIO_PIN_5) == GPIO_PIN_SET) {

If (currentMenu == MENU_TOP_MEASUREMENTS) currentMenu = MENU_MEASUREMENTS_1_1;

Else if (currentMenu == MENU_TOP_DATA_ENTRY) currentMenu = MENU_DATA_ENTRY_2_1;

Else if (currentMenu == MENU_TOP_DIAGNOSTICS) currentMenu = MENU_DIAGNOSTICS_3_1;

}

}

}

If (flag_S5) {

If ((HAL_GetTick() – LastTick_S5_PA9) > 50) {

Flag_S5 = 0;

If (HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_9) == GPIO_PIN_SET) {

// Top Menu Cycle

If (currentMenu == MENU_TOP_MEASUREMENTS) currentMenu = MENU_TOP_DATA_ENTRY;

Else if (currentMenu == MENU_TOP_DATA_ENTRY) currentMenu = MENU_TOP_DIAGNOSTICS;

Else if (currentMenu == MENU_TOP_DIAGNOSTICS) currentMenu = MENU_TOP_MEASUREMENTS;

// Measurement Sub-Menu Cycle

Else if (currentMenu == MENU_MEASUREMENTS_1_5) currentMenu = MENU_MEASUREMENTS_1_1;

Else if (currentMenu >= MENU_MEASUREMENTS_1_1 && currentMenu < MENU_MEASUREMENTS_1_5) currentMenu++;

// Data Entry Sub-Menu Cycle

Else if (currentMenu == MENU_DATA_ENTRY_2_3) currentMenu = MENU_DATA_ENTRY_2_1;

Else if (currentMenu >= MENU_DATA_ENTRY_2_1 && currentMenu < MENU_DATA_ENTRY_2_3) currentMenu++;

// Diagnostics Sub-Menu Cycle

Else if (currentMenu == MENU_DIAGNOSTICS_3_4) currentMenu = MENU_DIAGNOSTICS_3_1;

Else if (currentMenu >= MENU_DIAGNOSTICS_3_1 && currentMenu < MENU_DIAGNOSTICS_3_4) currentMenu++;

}

}

}

// --- KEYPAD TWO-STAGE DEBOUNCE & DATA ENTRY ---

If (flag_Keypad) {

If ((HAL_GetTick() – LastTick_Keypad) > 50) {

Flag_Keypad = 0;

Char key = read_keypad_matrix(active_column_pin);

If (key != ‘\0’) {

lastKeyPressed = key;

// ROUTE 1: Fuel Entry

If (currentMenu == MENU_DATA_ENTRY_2_1 && is_editing_data) {

If (lastKeyPressed >= ‘0’ && lastKeyPressed <= ‘9’) {

If (fuel_idx < 5) {

Fuel_input_str[fuel_idx] = lastKeyPressed;

Fuel_idx++;

Fuel_input_str[fuel_idx] = ‘\0’;

}

} else if (lastKeyPressed == ‘*’) {

Memset(fuel_input_str, 0, sizeof(fuel_input_str));

Fuel_idx = 0;

}

}

// ROUTE 2: Odometer Entry (Allow up to 6 digits)

Else if (currentMenu == MENU_DATA_ENTRY_2_2 && is_editing_odo) {

If (lastKeyPressed >= ‘0’ && lastKeyPressed <= ‘9’) {

If (odo_idx < 6) {

Odo_input_str[odo_idx] = lastKeyPressed;

Odo_idx++;

Odo_input_str[odo_idx] = ‘\0’;

}

} else if (lastKeyPressed == ‘*’) {

Memset(odo_input_str, 0, sizeof(odo_input_str));

Odo_idx = 0;

}

}

// ROUTE 3: Data Logging Toggle (No Edit mode needed, just press * or #)

Else if (currentMenu == MENU_DATA_ENTRY_2_3) {

If (lastKeyPressed == ‘*’) is_logging_enabled = 1; // ON

Else if (lastKeyPressed == ‘#’) is_logging_enabled = 0; // OFF

}

}

}

}

Static uint32_t lastSonarTick = 0;

// Ping the sensor every 100ms

If ((HAL_GetTick() – lastSonarTick) > 100) {

lastSonarTick = HAL_GetTick();

HCSR04_Trigger();

}

// --- 2. Alarm Checking & LED Control ---

If (alarmEnabled) {

// Check Physical Thresholds First

Proximity = (distance <= 10.0f && distance > 0.0f) ? 1 : 0;

High_temp = (current_temperature > 30.0f) ? 1 : 0;

// OVERRIDE them if the PC sent a SetWarn command!

If (force_prox) proximity = 1;

If (force_temp) high_temp = 1;

// Flash Alarms if active (500ms ON, 500ms OFF = 1Hz)

If (proximity){

FlashD2LEDS(GPIOB, GPIO_PIN_0, 2.0f); // Assuming D2 is PB0

} else {

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_RESET);

}

If (high_temp){

FlashD5LEDS(GPIOA, GPIO_PIN_0, 2.0f); // Assuming D5 is PA0

} else{

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_0, GPIO_PIN_RESET);

}

// D3 and D4 stay off during alarms unless their specific warnings are coded

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_4, GPIO_PIN_RESET);

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_1, GPIO_PIN_RESET);

} else {

// Normal Solid State Toggling

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, D2On ? GPIO_PIN_SET : GPIO_PIN_RESET);

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_4, D3On ? GPIO_PIN_SET : GPIO_PIN_RESET);

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_1, D4On ? GPIO_PIN_SET : GPIO_PIN_RESET);

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_0, D5On ? GPIO_PIN_SET : GPIO_PIN_RESET);

}

// --- 3. Process UART Telemetry Request ---

If (statRequested) {

statRequested = 0;

// Added ‘static’ so the memory stays alive!

// Added ‘static’ to ensure memory remains valid for the UART transmit [cite: 428]

Static char statMsg[512];

// Format the telemetry according to Table 3 and Section 4.6.2 [cite: 605, 598]

Sprintf(statMsg,

“@%s\n”                       // YYYY/MM/DD HH:MM:SS\n [cite: 605]

“Distance:    %04.1f cm\n”      // xxx.x cm (5 chars total: 3 digits, dot, 1 decimal)

“Temperature: %+04.1f C\n”   // xx.x C [cite: 605]

“Light:      %04.0f lux\n”       // xxxx lux

“X accel:     %+05.2f g\n”      // x.xx g (Leading + or – for accelerations) [cite: 605, 608]

“Y accel:     %+05.2f g\n”      // x.xx g [cite: 605]

“Z accel:     %+05.2f g\n”      // x.xx g [cite: 605]

“Unsafe driving:    %u\n”      // y (1 for True, 0 for False)

“Impact detected:   %u\n”     // y

“Low-Light warning: %u\n”   // y

“Proximity warning: %u\n”   // y

“High Temperature:  %u\n”    // y

“GPSlat:   %+010.5f\n”       // GPS formatting (supports negative values)

“GPSlong:  %+010.5f\n”      // GPS formatting

“&\n”,                      // Final termination

Datetime, distance, current_temperature, light,

X_accel, y_accel, z_accel,

Unsafe, impact, low_light, proximity, high_temp,

Gps_lat, gps_lon

);

HAL_UART_Transmit(&huart2, (uint8_t*)statMsg, strlen(statMsg), 1000);

}

Static uint32_t lastOledTick = 0;

// --- Process Fuel Efficiency Request ---

If (rfeRequested) {

rfeRequested = 0;

static char rfeMsg[128];

float km_per_l = 0.0f;

float l_per_100km = 0.0f;

// Prevent divide-by-zero errors!

If (fuel_liters > 0.0f && odo_km > 0.0f) {

Km_per_l = odo_km / fuel_liters;

L_per_100km = (fuel_liters / odo_km) * 100.0f;

}

// Format exactly as required by Table 4 [cite: 607]

Sprintf(rfeMsg,

“@%s\n”

“Fuel Eff: %04.1f km/L %04.1f L/100km\n”

“&\n”,

Datetime, km_per_l, l_per_100km

);

HAL_UART_Transmit(&huart2, (uint8_t*)rfeMsg, strlen(rfeMsg), 1000);

}

// Update the OLED every 200ms

If ((HAL_GetTick() – lastOledTick) > 200) {

lastOledTick = HAL_GetTick();

// --- 1. FETCH THE LIVE RTC TIME ---

RTC_TimeTypeDef sTime = {0};

RTC_DateTypeDef sDate = {0};

// CRITICAL: Must read Time first, then Date to unlock the hardware registers!

HAL_RTC_GetTime(&hrtc, &sTime, RTC_FORMAT_BIN);

HAL_RTC_GetDate(&hrtc, &sDate, RTC_FORMAT_BIN);

// Format it into our global string for both the OLED and UART to use

Sprintf(datetime, “20%02d/%02d/%02d %02d:%02d:%02d”,

sDate.Year, sDate.Month, sDate.Date,

sTime.Hours, sTime.Minutes, sTime.Seconds);

// ----------------------------------

Char displayStr[20];

Ssd1306_Fill(Black);

//  ALARM SCREEN HIJACKING ---

If (proximity) {

Ssd1306_SetCursor(0, 0);

Ssd1306_WriteString(“===== WARNING ====”, Font_7x10, White);

Ssd1306_SetCursor(0, 11);

Ssd1306_WriteString(“----Proximity-----“, Font_7x10, White);

Ssd1306_SetCursor(0, 22);

Sprintf(displayStr, “Dist: %04.1f cm”, distance);

Ssd1306_WriteString(displayStr, Font_7x10, White);

}

Else if (high_temp) {

Ssd1306_SetCursor(0, 0);

Ssd1306_WriteString(“===== WARNING ====”, Font_7x10, White);

Ssd1306_SetCursor(0, 11);

Ssd1306_WriteString(“-High Temperature-“, Font_7x10, White);

Ssd1306_SetCursor(0, 22);

Sprintf(displayStr, “Temp: %+04.1f C”, current_temperature);

Ssd1306_WriteString(displayStr, Font_7x10, White);

}

Else {

// Row 1: Print the Live Date/Time (cutting off the seconds to fit the screen)

Ssd1306_SetCursor(0, 0);

Char shortTime[20];

Sprintf(shortTime, “20%02d/%02d/%02d %02d:%02d”,

sDate.Year, sDate.Month, sDate.Date, sTime.Hours, sTime.Minutes);

ssd1306_WriteString(shortTime, Font_7x10, White);

// Row 2 & 3: Depend on the current menu state [cite: 752]

Switch(currentMenu) {

Case MENU_TOP_MEASUREMENTS:

Ssd1306_SetCursor(0, 11);

Ssd1306_WriteString(“== Measurements ==”, Font_7x10, White);

Ssd1306_SetCursor(0, 22);

Ssd1306_WriteString(“Press -> display”, Font_7x10, White);

Break;

Case MENU_MEASUREMENTS_1_1:

Ssd1306_SetCursor(0, 11);

Sprintf(displayStr, “Dist: %04.1f cm”, distance);

Ssd1306_WriteString(displayStr, Font_7x10, White);

Ssd1306_SetCursor(0, 22);

Sprintf(displayStr, “Temp: %+04.1f C”, current_temperature);

Ssd1306_WriteString(displayStr, Font_7x10, White);

Break;

Case MENU_MEASUREMENTS_1_2:

Ssd1306_SetCursor(0, 11);

Sprintf(displayStr, “Accel: %+04.2f g”, x_accel); // Defaulting to X accel for display

Ssd1306_WriteString(displayStr, Font_7x10, White);

Ssd1306_SetCursor(0, 22);

Sprintf(displayStr, “Light: %04.0f lux”, light);

Ssd1306_WriteString(displayStr, Font_7x10, White);

Break;

Case MENU_MEASUREMENTS_1_3:

Ssd1306_SetCursor(0, 11);

Sprintf(displayStr, “Lat: %+010.5f”, gps_lat);

Ssd1306_WriteString(displayStr, Font_7x10, White);

Ssd1306_SetCursor(0, 22);

Sprintf(displayStr, “Long: %+010.5f”, gps_lon);

Ssd1306_WriteString(displayStr, Font_7x10, White);

Break;

Case MENU_MEASUREMENTS_1_4:

Ssd1306_SetCursor(0, 11);

Ssd1306_WriteString(“Heading: 000 deg”, Font_7x10, White);

Ssd1306_SetCursor(0, 22);

Ssd1306_WriteString(“Speed: 000.0 km/h”, Font_7x10, White);

Break;

Case MENU_MEASUREMENTS_1_5:

Ssd1306_SetCursor(0, 11);

Ssd1306_WriteString(“Fuel Efficiency:”, Font_7x10, White);

Ssd1306_SetCursor(0, 22);

Float disp_kml = (fuel_liters > 0) ? (odo_km / fuel_liters) : 0.0f;

Float disp_l100 = (odo_km > 0) ? ((fuel_liters / odo_km) * 100.0f) : 0.0f;

Sprintf(displayStr, “%04.1f k/L %04.1f L/100”, disp_kml, disp_l100);

Ssd1306_WriteString(displayStr, Font_7x10, White);

Break;

Case MENU_TOP_DATA_ENTRY:

Ssd1306_SetCursor(0, 11);

Ssd1306_WriteString(“=== Data Entry ===”, Font_7x10, White);

Ssd1306_SetCursor(0, 22);

Ssd1306_WriteString(“Press -> display”, Font_7x10, White);

Break;

Case MENU_DATA_ENTRY_2_1:

Ssd1306_SetCursor(0, 0);

Ssd1306_WriteString(“== Enter Fuel ==”, Font_7x10, White);

Ssd1306_SetCursor(0, 15);

If (is_editing_data) {

Char displayBuf[20];

Sprintf(displayBuf, “> %s_”, fuel_input_str);

Ssd1306_WriteString(displayBuf, Font_7x10, White);

} else {

Ssd1306_WriteString(“Press S3 to Edit”, Font_7x10, White);

}

Break;

Case MENU_DATA_ENTRY_2_2:

Ssd1306_SetCursor(0, 0);

Ssd1306_WriteString(“== Enter Odo ==”, Font_7x10, White);

Ssd1306_SetCursor(0, 15);

If (is_editing_odo) {

Char displayBuf[20];

Sprintf(displayBuf, “> %s_ km”, odo_input_str);

Ssd1306_WriteString(displayBuf, Font_7x10, White);

} else {

Ssd1306_WriteString(“Press S3 to Edit”, Font_7x10, White);

}

Break;

Case MENU_DATA_ENTRY_2_3:

Ssd1306_SetCursor(0, 0);

Ssd1306_WriteString(“== Data Log ==”, Font_7x10, White);

Ssd1306_SetCursor(0, 15);

If (is_logging_enabled) {

Ssd1306_WriteString(“Status: [ ON ]”, Font_7x10, White);

} else {

Ssd1306_WriteString(“Status: [ OFF ]”, Font_7x10, White);

}

Ssd1306_SetCursor(0, 25);

Ssd1306_WriteString(“*=ON  #=OFF”, Font_7x10, White);

Break;

Case MENU_TOP_DIAGNOSTICS:

Ssd1306_SetCursor(0, 11);

Ssd1306_WriteString(“== Diagnostics ===”, Font_7x10, White);

Ssd1306_SetCursor(0, 22);

Ssd1306_WriteString(“Press -> display”, Font_7x10, White);

Break;

Case MENU_DIAGNOSTICS_3_1:

Ssd1306_SetCursor(0, 11);

Ssd1306_WriteString(“Diagnostics”, Font_7x10, White);

Ssd1306_SetCursor(0, 22);

Ssd1306_WriteString(“SD-card: OK”, Font_7x10, White); // Hardcoded for demo 3

Break;

Case MENU_DIAGNOSTICS_3_2:

Ssd1306_SetCursor(0, 11);

Ssd1306_WriteString(“Diagnostics”, Font_7x10, White);

Ssd1306_SetCursor(0, 22);

Ssd1306_WriteString(“MPU-6050: NOT OK”, Font_7x10, White); // Hardcoded

Break;

Case MENU_DIAGNOSTICS_3_3:

Ssd1306_SetCursor(0, 11);

Ssd1306_WriteString(“Diagnostics”, Font_7x10, White);

Ssd1306_SetCursor(0, 22);

Ssd1306_WriteString(“GPS: NOT OK”, Font_7x10, White); // Hardcoded

Break;

Case MENU_DIAGNOSTICS_3_4:

Ssd1306_SetCursor(0, 11);

Ssd1306_WriteString(“Diagnostics”, Font_7x10, White);

Ssd1306_SetCursor(0, 22);

If (is_logging_enabled) {

Ssd1306_WriteString(“Log Data: ENABLED”, Font_7x10, White);

} else {

Ssd1306_WriteString(“Log Data: DISABLED”, Font_7x10, White);

}

Break;

Default:

// Catch-all for menus we haven’t built out yet

Ssd1306_SetCursor(0, 11);

Ssd1306_WriteString(“Under Construct.”, Font_7x10, White);

Break;

}

}

Ssd1306_UpdateScreen();

}

}

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

/* USER CODE END 3 */

}

/**

* @brief System Clock Configuration

* @retval None

*/

Void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Configure the main internal regulator output voltage

*/

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI|RCC_OSCILLATORTYPE_LSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.LSIState = RCC_LSI_ON;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON;

RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSI;

RCC_OscInitStruct.PLL.PLLM = 16;

RCC_OscInitStruct.PLL.PLLN = 336;

RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV4;

RCC_OscInitStruct.PLL.PLLQ = 4;

If (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV2;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;

If (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_2) != HAL_OK)

{

Error_Handler();

}

}

/**

* @brief I2C1 Initialization Function

* @param None

* @retval None

*/

Static void MX_I2C1_Init(void)

{

/* USER CODE BEGIN I2C1_Init 0 */

/* USER CODE END I2C1_Init 0 */

/* USER CODE BEGIN I2C1_Init 1 */

/* USER CODE END I2C1_Init 1 */

Hi2c1.Instance = I2C1;

Hi2c1.Init.ClockSpeed = 400000;

Hi2c1.Init.DutyCycle = I2C_DUTYCYCLE_2;

Hi2c1.Init.OwnAddress1 = 0;

Hi2c1.Init.AddressingMode = I2C_ADDRESSINGMODE_7BIT;

Hi2c1.Init.DualAddressMode = I2C_DUALADDRESS_DISABLE;

Hi2c1.Init.OwnAddress2 = 0;

Hi2c1.Init.GeneralCallMode = I2C_GENERALCALL_DISABLE;

Hi2c1.Init.NoStretchMode = I2C_NOSTRETCH_DISABLE;

If (HAL_I2C_Init(&hi2c1) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN I2C1_Init 2 */

/* USER CODE END I2C1_Init 2 */

}

/**

* @brief RTC Initialization Function

* @param None

* @retval None

*/

Static void MX_RTC_Init(void)

{

/* USER CODE BEGIN RTC_Init 0 */

/* USER CODE END RTC_Init 0 */

RTC_TimeTypeDef sTime = {0};

RTC_DateTypeDef sDate = {0};

/* USER CODE BEGIN RTC_Init 1 */

/* USER CODE END RTC_Init 1 */

/** Initialize RTC Only

*/

Hrtc.Instance = RTC;

Hrtc.Init.HourFormat = RTC_HOURFORMAT_24;

Hrtc.Init.AsynchPrediv = 127;

Hrtc.Init.SynchPrediv = 255;

Hrtc.Init.OutPut = RTC_OUTPUT_DISABLE;

Hrtc.Init.OutPutPolarity = RTC_OUTPUT_POLARITY_HIGH;

Hrtc.Init.OutPutType = RTC_OUTPUT_TYPE_OPENDRAIN;

If (HAL_RTC_Init(&hrtc) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN Check_RTC_BKUP */

/* USER CODE END Check_RTC_BKUP */

/** Initialize RTC and set the Time and Date

*/

sTime.Hours = 0x0;

sTime.Minutes = 0x0;

sTime.Seconds = 0x0;

sTime.DayLightSaving = RTC_DAYLIGHTSAVING_NONE;

sTime.StoreOperation = RTC_STOREOPERATION_RESET;

if (HAL_RTC_SetTime(&hrtc, &sTime, RTC_FORMAT_BCD) != HAL_OK)

{

Error_Handler();

}

sDate.WeekDay = RTC_WEEKDAY_MONDAY;

sDate.Month = RTC_MONTH_JANUARY;

sDate.Date = 0x1;

sDate.Year = 0x0;

if (HAL_RTC_SetDate(&hrtc, &sDate, RTC_FORMAT_BCD) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN RTC_Init 2 */

/* USER CODE END RTC_Init 2 */

}

/**

* @brief TIM3 Initialization Function

* @param None

* @retval None

*/

Static void MX_TIM3_Init(void)

{

/* USER CODE BEGIN TIM3_Init 0 */

/* USER CODE END TIM3_Init 0 */

TIM_ClockConfigTypeDef sClockSourceConfig = {0};

TIM_MasterConfigTypeDef sMasterConfig = {0};

TIM_IC_InitTypeDef sConfigIC = {0};

/* USER CODE BEGIN TIM3_Init 1 */

/* USER CODE END TIM3_Init 1 */

Htim3.Instance = TIM3;

Htim3.Init.Prescaler = 83;

Htim3.Init.CounterMode = TIM_COUNTERMODE_UP;

Htim3.Init.Period = 65535;

Htim3.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1;

Htim3.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_DISABLE;

If (HAL_TIM_Base_Init(&htim3) != HAL_OK)

{

Error_Handler();

}

sClockSourceConfig.ClockSource = TIM_CLOCKSOURCE_INTERNAL;

if (HAL_TIM_ConfigClockSource(&htim3, &sClockSourceConfig) != HAL_OK)

{

Error_Handler();

}

If (HAL_TIM_IC_Init(&htim3) != HAL_OK)

{

Error_Handler();

}

sMasterConfig.MasterOutputTrigger = TIM_TRGO_RESET;

sMasterConfig.MasterSlaveMode = TIM_MASTERSLAVEMODE_DISABLE;

if (HAL_TIMEx_MasterConfigSynchronization(&htim3, &sMasterConfig) != HAL_OK)

{

Error_Handler();

}

sConfigIC.ICPolarity = TIM_INPUTCHANNELPOLARITY_RISING;

sConfigIC.ICSelection = TIM_ICSELECTION_DIRECTTI;

sConfigIC.ICPrescaler = TIM_ICPSC_DIV1;

sConfigIC.ICFilter = 0;

if (HAL_TIM_IC_ConfigChannel(&htim3, &sConfigIC, TIM_CHANNEL_1) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN TIM3_Init 2 */

/* USER CODE END TIM3_Init 2 */

}

/**

* @brief TIM4 Initialization Function

* @param None

* @retval None

*/

Static void MX_TIM4_Init(void)

{

/* USER CODE BEGIN TIM4_Init 0 */

/* USER CODE END TIM4_Init 0 */

TIM_MasterConfigTypeDef sMasterConfig = {0};

TIM_IC_InitTypeDef sConfigIC = {0};

/* USER CODE BEGIN TIM4_Init 1 */

/* USER CODE END TIM4_Init 1 */

Htim4.Instance = TIM4;

Htim4.Init.Prescaler = 0;

Htim4.Init.CounterMode = TIM_COUNTERMODE_UP;

Htim4.Init.Period = 65535;

Htim4.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1;

Htim4.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_DISABLE;

If (HAL_TIM_IC_Init(&htim4) != HAL_OK)

{

Error_Handler();

}

sMasterConfig.MasterOutputTrigger = TIM_TRGO_RESET;

sMasterConfig.MasterSlaveMode = TIM_MASTERSLAVEMODE_DISABLE;

if (HAL_TIMEx_MasterConfigSynchronization(&htim4, &sMasterConfig) != HAL_OK)

{

Error_Handler();

}

sConfigIC.ICPolarity = TIM_INPUTCHANNELPOLARITY_RISING;

sConfigIC.ICSelection = TIM_ICSELECTION_DIRECTTI;

sConfigIC.ICPrescaler = TIM_ICPSC_DIV1;

sConfigIC.ICFilter = 0;

if (HAL_TIM_IC_ConfigChannel(&htim4, &sConfigIC, TIM_CHANNEL_1) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN TIM4_Init 2 */

/* USER CODE END TIM4_Init 2 */

}

/**

* @brief USART2 Initialization Function

* @param None

* @retval None

*/

Static void MX_USART2_UART_Init(void)

{

/* USER CODE BEGIN USART2_Init 0 */

/* USER CODE END USART2_Init 0 */

/* USER CODE BEGIN USART2_Init 1 */

/* USER CODE END USART2_Init 1 */

Huart2.Instance = USART2;

Huart2.Init.BaudRate = 57600;

Huart2.Init.WordLength = UART_WORDLENGTH_9B;

Huart2.Init.StopBits = UART_STOPBITS_1;

Huart2.Init.Parity = UART_PARITY_EVEN;

Huart2.Init.Mode = UART_MODE_TX_RX;

Huart2.Init.HwFlowCtl = UART_HWCONTROL_NONE;

Huart2.Init.OverSampling = UART_OVERSAMPLING_16;

If (HAL_UART_Init(&huart2) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN USART2_Init 2 */

/* USER CODE END USART2_Init 2 */

}

/**

* @brief GPIO Initialization Function

* @param None

* @retval None

*/

Static void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* USER CODE BEGIN MX_GPIO_Init_1 */

/* USER CODE END MX_GPIO_Init_1 */

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOH_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

__HAL_RCC_GPIOB_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(UltraSonic_Trigger_pin_GPIO_Port, UltraSonic_Trigger_pin_Pin, GPIO_PIN_RESET);

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOC, Row_1_Pin|Row_2_Pin|Row_3_Pin|Row_4_Pin, GPIO_PIN_SET);

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOA, LED_D5_Pin|LED_D4_Pin|LED_D3_Pin|LD2_Pin, GPIO_PIN_RESET);

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(LED_D2_GPIO_Port, LED_D2_Pin, GPIO_PIN_RESET);

/*Configure GPIO pins : UltraSonic_Trigger_pin_Pin Row_1_Pin Row_2_Pin Row_3_Pin

Row_4_Pin */

GPIO_InitStruct.Pin = UltraSonic_Trigger_pin_Pin|Row_1_Pin|Row_2_Pin|Row_3_Pin

|Row_4_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

/*Configure GPIO pins : LED_D5_Pin LED_D4_Pin LED_D3_Pin LD2_Pin */

GPIO_InitStruct.Pin = LED_D5_Pin|LED_D4_Pin|LED_D3_Pin|LD2_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

/*Configure GPIO pin : LED_D2_Pin */

GPIO_InitStruct.Pin = LED_D2_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(LED_D2_GPIO_Port, &GPIO_InitStruct);

/*Configure GPIO pins : Top_Button_Pin Column_1_Pin Column_2_Pin Column_3_Pin

Center_Button_Pin Right_Button_Pin */

GPIO_InitStruct.Pin = Top_Button_Pin|Column_1_Pin|Column_2_Pin|Column_3_Pin

|Center_Button_Pin|Right_Button_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_IT_RISING;

GPIO_InitStruct.Pull = GPIO_PULLDOWN;

HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);

/*Configure GPIO pins : Left_Button_Pin Bottom_Button_Pin */

GPIO_InitStruct.Pin = Left_Button_Pin|Bottom_Button_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_IT_RISING;

GPIO_InitStruct.Pull = GPIO_PULLDOWN;

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

/* EXTI interrupt init*/

HAL_NVIC_SetPriority(EXTI4_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(EXTI4_IRQn);

HAL_NVIC_SetPriority(EXTI9_5_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(EXTI9_5_IRQn);

HAL_NVIC_SetPriority(EXTI15_10_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(EXTI15_10_IRQn);

/* USER CODE BEGIN MX_GPIO_Init_2 */

/* USER CODE END MX_GPIO_Init_2 */

}

/* USER CODE BEGIN 4 */

Void FlashD2LEDS(GPIO_TypeDef *port1, uint16_t pin1, float frequency)

{

Static uint32_t previousTick = 0;

Uint32_t interval = (uint32_t)(1000.0f / (frequency));

If (HAL_GetTick() – previousTick >= interval)

{

previousTick = HAL_GetTick();

HAL_GPIO_TogglePin(port1, pin1);

}

}

Void FlashD5LEDS(GPIO_TypeDef *port2, uint16_t pin2, float frequency)

{

Static uint32_t previousTick = 0;

Uint32_t interval = (uint32_t)(1000.0f / (frequency));

If (HAL_GetTick() – previousTick >= interval)

{

previousTick = HAL_GetTick();

HAL_GPIO_TogglePin(port2, pin2);

}

}

Char read_keypad_matrix(uint16_t column_pin) {

// Standard 4x3 keypad layout

Char key_map[4][3] = {

{‘1’, ‘2’, ‘3’},

{‘4’, ‘5’, ‘6’},

{‘7’, ‘8’, ‘9’},

{‘*’, ‘0’, ‘#’}

};

Int col = -1;

Int row = -1;

// 1. Determine which column triggered the interrupt

If (column_pin == GPIO_PIN_13) col = 0;

Else if (column_pin == GPIO_PIN_14) col = 1;

Else if (column_pin == GPIO_PIN_15) col = 2;

If (col == -1) return ‘\0’;

// 2. Set all rows LOW

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_1 | GPIO_PIN_2 | GPIO_PIN_3 | GPIO_PIN_4, GPIO_PIN_RESET);

// A tiny hardware delay to let the voltage physically travel through the membrane

#define MATRIX_DELAY() for(volatile int d=0; d<1000; d++) {}

// 3. Scan Row 1

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_1, GPIO_PIN_SET);

MATRIX_DELAY();

If (HAL_GPIO_ReadPin(GPIOB, column_pin) == GPIO_PIN_SET) row = 0;

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_1, GPIO_PIN_RESET);

// 4. Scan Row 2

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_2, GPIO_PIN_SET);

MATRIX_DELAY();

If (HAL_GPIO_ReadPin(GPIOB, column_pin) == GPIO_PIN_SET) row = 1;

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_2, GPIO_PIN_RESET);

// 5. Scan Row 3

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_3, GPIO_PIN_SET);

MATRIX_DELAY();

If (HAL_GPIO_ReadPin(GPIOB, column_pin) == GPIO_PIN_SET) row = 2;

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_3, GPIO_PIN_RESET);

// 6. Scan Row 4

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_4, GPIO_PIN_SET);

MATRIX_DELAY();

If (HAL_GPIO_ReadPin(GPIOB, column_pin) == GPIO_PIN_SET) row = 3;

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_4, GPIO_PIN_RESET);

// 7. RESTORE all rows HIGH so the next press triggers the interrupt!

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_1 | GPIO_PIN_2 | GPIO_PIN_3 | GPIO_PIN_4, GPIO_PIN_SET);

// 8. Return the mapped key if we found a match

If (row != -1) {

Return key_map[row][col];

}

Return ‘\0’;

}

Void HAL_GPIO_EXTI_Callback(uint16_t GPIO_Pin)

{

CurrentTick = HAL_GetTick();

// TOP BUTTON (UP-S1) on PB10 -> Toggles LED D2

If(GPIO_Pin == GPIO_PIN_10) {

If ((CurrentTick – LastTick_S1_PB10) > 200) {

LastTick_S1_PB10 = CurrentTick;

Flag_S1 = 1;

}

}

// LEFT BUTTON (LEFT-S2) on PA8 -> Toggles LED D3

Else if(GPIO_Pin == GPIO_PIN_8) {

If ((CurrentTick – LastTick_S2_PA8) > 200) {

LastTick_S2_PA8 = CurrentTick;

Flag_S2 = 1; // Signal main loop to toggle D3

}

}

// MIDDLE BUTTON (MIDDLE-S3) on PB4 -> UART Transmit

Else if(GPIO_Pin == GPIO_PIN_4) {

If ((CurrentTick – LastTick_S3_PB4) > 200) {

LastTick_S3_PB4 = CurrentTick;

Flag_S3 = 1;

}

}

// RIGHT BUTTON (RIGHT-S4) on PB5 -> Toggles LED D4

Else if(GPIO_Pin == GPIO_PIN_5) {

If ((CurrentTick – LastTick_S4_PB5) > 200) {

LastTick_S4_PB5 = CurrentTick;

Flag_S4 = 1;

}

}

// BOTTOM BUTTON (DOWN-S5) on PA9 -> Toggles LED D5

Else if(GPIO_Pin == GPIO_PIN_9) {

If ((CurrentTick – LastTick_S5_PA9) > 200) {

LastTick_S5_PA9 = CurrentTick;

Flag_S5 = 1;

}

}

// KEYPAD COLUMNS on PB13, PB14, PB15

Else if(GPIO_Pin == GPIO_PIN_13 || GPIO_Pin == GPIO_PIN_14 || GPIO_Pin == GPIO_PIN_15) {

If ((CurrentTick – LastTick_Keypad) > 100) {

LastTick_Keypad = CurrentTick;

Active_column_pin = GPIO_Pin; // Remember WHICH column was hit

Flag_Keypad = 1; // Tell the main loop to start the 50ms timer

}

}

}

// Automatically called by HAL every time PB6 sees a pulse!

// This single callback handles BOTH the Temperature and Ultrasonic sensors!

Void HAL_TIM_IC_CaptureCallback(TIM_HandleTypeDef *htim)

{

// ---------------------------------------------------------

// 1. Temperature Sensor (LMT01) on TIM4

// ---------------------------------------------------------

If (htim->Instance == TIM4 && htim->Channel == HAL_TIM_ACTIVE_CHANNEL_1) {

Uint32_t current_time = HAL_GetTick();

// If it has been more than 4ms since the last pulse, a NEW burst is starting!

If ((current_time – last_capture_time) > 4) {

// If we counted pulses in the previous burst, calculate the math

If (pulse_count > 0) {

Current_temperature = ((float)pulse_count / 4096.0f * 256.0f) – 50.0f;

Temp_ready = 1;

}

// Reset the counter to 0 for this new incoming burst

Pulse_count = 0;

}

// Add +1 to our count, and mark the time this pulse hit

Pulse_count++;

Last_capture_time = current_time;

}

// ---------------------------------------------------------

// 2. Ultrasonic Sensor (HC-SR04) Echo on TIM3

// ---------------------------------------------------------

If (htim->Instance == TIM3 && htim->Channel == HAL_TIM_ACTIVE_CHANNEL_1) {

If (Is_First_Captured == 0) {

// Read the first timestamp (Rising Edge)

IC_Val1 = HAL_TIM_ReadCapturedValue(htim, TIM_CHANNEL_1);

Is_First_Captured = 1;

// Switch the timer to look for the Falling Edge now

__HAL_TIM_SET_CAPTUREPOLARITY(htim, TIM_CHANNEL_1, TIM_INPUTCHANNELPOLARITY_FALLING);

}

Else if (Is_First_Captured == 1) {

// Read the second timestamp (Falling Edge)

IC_Val2 = HAL_TIM_ReadCapturedValue(htim, TIM_CHANNEL_1);

__HAL_TIM_SET_COUNTER(htim, 0); // Reset counter

// Calculate the width of the echo pulse

Uint32_t difference;

If (IC_Val2 > IC_Val1) {

Difference = IC_Val2 – IC_Val1;

} else {

// Account for 16-bit timer rollover

Difference = (0xFFFF – IC_Val1) + IC_Val2;

}

// Convert microseconds to centimeters

Distance = (float)difference / 58.0f;

// Reset flags and switch back to Rising Edge for the next ping

Is_First_Captured = 0;

__HAL_TIM_SET_CAPTUREPOLARITY(htim, TIM_CHANNEL_1, TIM_INPUTCHANNELPOLARITY_RISING);

// Stop listening until we trigger the sensor again in the main loop

HAL_TIM_IC_Stop_IT(&htim3, TIM_CHANNEL_1);

}

}

}

Void HAL_UART_RxCpltCallback(UART_HandleTypeDef *huart)

{

If (huart->Instance == USART2) {

// Clean the byte (mask parity) before checking it

Uint8_t cleanByte = rxByte & 0x7F;

rxBuffer[rxIndex] = cleanByte;

rxIndex++;

rxBuffer[rxIndex] = ‘\0’;

// Check if we hit an “Enter” key (\n or \r) or filled the buffer

If (cleanByte == ‘\n’ || cleanByte == ‘\r’ || rxIndex >= sizeof(rxBuffer) – 1) {

// 1. Status Request

If (strstr((char*)rxBuffer, “@Stat&”) != NULL) {

statRequested = 1;

}

// 2. Request Fuel Efficiency [cite: 606]

Else if (strstr((char*)rxBuffer, “@RFE&”) != NULL) {

rfeRequested = 1;

}

// 3. Set Fuel Data

Else if (strstr((char*)rxBuffer, “@SFD”) != NULL) {

// 1. Find where the numbers start (5 characters in, right after “@SFD “)

Char *firstNum = strstr((char*)rxBuffer, “@SFD “) + 5;

// 2. Find the semicolon that separates the two numbers

Char *semicolon = strchr(firstNum, ‘;’);

If (firstNum != NULL && semicolon != NULL) {

// atof() converts a string to a float!

// It automatically stops reading when it hits the ‘;’

Fuel_liters = atof(firstNum);

// Start reading the second float one space after the ‘;’

Odo_km = atof(semicolon + 1);

}

}

// 4. Set Warning Overrides

Else if (strstr((char*)rxBuffer, “@SetWarn”) != NULL) {

// Command format is strictly: @SetWarn C=Y&

// We just grab the characters at index 9 and 11, and subtract ‘0’ to make them real integers!

Int c_val = rxBuffer[9] – ‘0’;

Int y_val = rxBuffer[11] – ‘0’;

If (c_val == 4) force_prox = y_val; // Proximity

If (c_val == 5) force_temp = y_val; // High Temp

}

rxIndex = 0; // Reset buffer for the next command

}

// Re-arm the interrupt

HAL_UART_Receive_IT(&huart2, &rxByte, 1);

}

}

Void HCSR04_Trigger(void)

{

// Set PC0 HIGH

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_0, GPIO_PIN_SET);

// Quick hardware loop to delay for ~10us at 84MHz

For (volatile int I = 0; I < 200; i++);

// Set PC0 LOW

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_0, GPIO_PIN_RESET);

// Enable TIM3 Input Capture interrupt to listen for the echo on PA6

HAL_TIM_IC_Start_IT(&htim3, TIM_CHANNEL_1);

}

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

Void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

While (1)

{

}

/* USER CODE END Error_Handler_Debug */

}

#ifdef USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

Void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

Ex: printf(“Wrong parameters value: file %s on line %d\r\n”, file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */